ICS 103 – Computer Programming in C

Lab #12: Strings

Objective:

Practice how to use Strings.

String Declaration & Initialization

        A string is a list of characters enclosed in double quotes. It is represented in C as an array of characters

        However, one additional cell is required for storing the terminating character, '\0',called NULL

        Example:  Even though the string "I like C" has 8 characters, we need to declare an array of size at least equal to 9.

# char str[]="I like C";

| 'I' | ' ' | 'l' | 'i' | 'k' | 'e' | ' ' | 'C' | '\0' |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that in the example above, the null character is automatically inserted by the compiler.  You can use

printf("%d",sizeof(str));

to know the size reserved for array str by the compiler  (9 in this case).

        All the string functions in C require the presence of the NULL character to work correctly.

         Strings can also be declared without specifying the size as in:

char str[]="I like C";

In this case, sufficient storage is allocated including that for NULL

        Like array, the individual characters of a string can be accessed by specifying the index.  Example,  assuming the above assignment, str[4] is the character 'k'

String I/O

        The easiest way to input string is by using the gets function

# Example:	char name[81];

# printf("Input your name");

# gets(name);

gets allows you to read a string that could include blanks.  It reads all characters including blanks up to pressing the "ENTER" key.  The "ENTER" key ('\n') character will not be part of the string.

        Also the easiest way to output string is by using the puts function as in:

# puts(name);

puts(name)  is equivalent to printf("%s\n",name)

        For scanning  from file, we have fgets(str, n, file) which expects 3 arguments: string variable,  number of characters to scan and the file to scan from.  If end of line ('\n') is encountered before scanning n-1 characters, fgets will stop reading otherwise it reads n-1 characters.

        Also we can print into a file using fputs(str, file).

        You can also use the scanf/printf functions for string I/O using the %s format specifier.

        However,  scanf terminates reading on encountering the blank character, so first and last names must be read separately:

# char first[10],last[10],full[20];

# printf("\nEnter your first name: ");

# scanf("%s",first);

# printf("\nEnter your last name: ");

# scanf("%s",last);

# printf("\nYour full name is : %s %s",first,last);

        Note that the address of operator, &, is not used in the scanf function.

Built-in string functions:

Built-in string processing functions can be accessed through the header file, string.h .  Some of these are:

        strcat(string1, string2)  appends strings2 to string1

        strcpy(string1, string2)  replaces string1 with string2

        strlen(string1)  returns the length of string1 excluding NULL character

        strchr(str, ch)  searches for the occurrence of character ch in the string str

        strstr (string1, string2)  searches for the occurrence of string2 in string1

- strtok(str, delimiters)  splits string str into parts (tokens) based on the delimiters

        strcmp(string1, string2)  compares string1 and string2 and  returns:

o       Negative integer if string1 < string2

o       0 if string1 = = string2

o       positive integer if string1 > string2.

The following program illustrates the use of some string functions:

# #include <string.h>

# #include <stdio.h>

# main() {

# char s1[81], s2[81];

# gets(s1);

# gets(s2);

# printf("lengths: %d  %d\n", strlen(s1),  strlen(s2));

# if (strcmp(s1, s2)==0)

# printf("The strings are equal\n");

# strcat(s1,s2);

# printf("%s\n",s1);

# return 0;

# }

Note: It is important to remember that strcmp() returns 0 if the strings are equal.

Built-in character functions:

        Built-in character functions can be accessed through the header file, ctype.h

        Some of the common functions are:

        isupper(ch)  returns nonzero if ch is uppercase, returns 0 otherwise

        islower(ch)  returns nonzero if ch is lowercase, returns 0 otherwise

        isalpha(ch)  returns nonzero if ch is an alphabet (lower or upper case) or 0

otherwise

        isalnum(ch)  returns nonzero if ch is an alphabet (lower or upper case) or a

digit (0,1,..9), or 0 otherwise

	isdigit(ch)  returns nonzero if ch is a digit (0, 1, …, 9) or 0 otherwise

        tolower(ch)  converts a character to its lower case if it has one

        toupper(ch)  converts a character to its upper case if it has one

Exercises:

#### Exercise 1:

Write a program that reads some text from the user, then calls a function called modify that you need to write.

This function receives a string and does the following changes:

-Each capital letter is converted to lower case

- Each lower case letter is converted to capital letter

-Each digit is incremented by 1, except 9 which becomes 0.

-Each other character is replaced by #

After calling the function modify, your program displays the modified text.

#### Exercise 2:

Write a program that reads a sentence from the user and counts how many words are in the sentence. You program displays the words and also displays their count as shown below:

It also puts the words in a new string without the punctuation characters. The words will be separated by a space.  After that, the new string containing words only is displayed.

Note : Use strtok and strcat functions. The delimiter string contains in addition to space, the punctuation characters.

#### Exercise 3:

The following is an implementation of the function strcmp.

int strcmp (char s1[], char s2[]) {

int i;

for (i=0; s1[i]!='\0'&&s2[i]!='\0'; i++ ) {

if(s1[i]!=s2[i])

return s1[i]-s2[i];

}

return s1[i]-s2[i];

}

As shown in the second run, the function strcmp is case sensitive.  You need to modify strcmp and call it my_strcmp so that it is case insensitive.  Write a main function to test it as shown below.